RaspPi Lab 4 – Using AI to Control the Motor

What you just did is called “hard-coding” the behavior of the motor. It runs on its own, but it always does the same thing every time no matter the environment around it. What if we wanted a way to change the behavior of the motor without changing the code that is running it? And what if we wanted to change that behavior with just how we move or turn our hands?

Step 1) If you haven’t already, go ahead and plug the webcam into your Raspberry Pi.

Step 2) This time you want to go into the ai-motor.c file. It’s in the same folder as the others have been.

Step 3) Similar to in Section 1 of this project, you need to set the pin modes for the motor in the void setup(void) function. You can put the lines you had from before the set your motor pin modes starting at line 31 of ai-motor.c.

Step 4) Now in the void loop(void) function we will be doing most of the work. I’ve given you two variables to work with at the top of this function, xval and speed, but you can add more if you want to eventually. You’ll also see a new function called get_handstate(fp, &lHand, &rHand) in this function. That function is how you get information from the machine learning algorithm about what state the hand is in. Don’t worry about the fp part, but lHand and rHand actually refer to the left and right hand respectively.

Step 5) At the very bottom of this file is a printf line. That’s going to print some of the info you can get about the hands from the machine learning algorithm. Go ahead and compile and run this project and you can actually see some of this info before you try using it to control the motor.

- Save and close this file

- Enter make ai-motor in the terminal

- Enter ./ai-motor and a window will open up with the webcam’s view. After a couple of seconds some stuff may start to print to the terminal too. If you hold up your left hand and move it around, you will see the info changes.

- Now close this and open the file back up. In the printf() function at the bottom you can change theta in both places it appears to instead be mag. So for instance it would be the following:

printf("INFO: Analog joystick: mag == %4d, speed = %4d\n", lHand.mag, speed);

- Once you’ve done that, try saving running it again and seeing what happens. What was theta telling you before about the left hand? What is mag telling you instead?

- Instead of mag, you also have the option to enter x or y where theta originally was. Here’s a chart that you can fill in with info on what each of these is telling you about the hand:

| lHand.theta | The angle of rotation of your left hand |
| --- | --- |
| lHand.mag |  |
| lHand.x |  |
| lHand.y |  |
| rHand.theta | The angle of rotation of your ________ hand |
| rHand.mag |  |
| rHand.x |  |
| rHand.y |  |

Step 6) So now you know how to get info about the hand. You also know how to control the motor. With this you can now control the motor using just the gestures of your hands. But, here are some helpful things to know:

- The maximum value for the motor speed is 1024. That means that if, for instance, you wanted to use the angle of your right hand to control the speed of the motor, you would need to do some math to make that angle [-180, 180] something in the range of [0, 1024].

- The pins that control the motor direction always have to be opposites (if one is HIGH the other must be LOW), and also they can only be HIGH or LOW. So you’ll need to use an if-statement to control the direction with your hands if you want to.

  - If-statements test if a condition is true, then only execute the code inside of them if that condition is true. So for example if I wrote…

—--------------------------------

if (2 > 1) {

printf(“Howdy!”);

}

—--------------------------------

then we would see Howdy get printed to the terminal because 2 is greater than 1. However, if instead I had…

—--------------------------------

if (2 < 1) {

printf(“Howdy!”);

}

—--------------------------------

then we wouldn’t see anything because 2 is not less than 1. The printf() statement never got executed!

  - We also have something called an else-statement, which we can have happen instead of our if-statement when it doesn’t meet the condition. You can read these as “if A is true, then do X, else do Y.”

—--------------------------------

if (A) {

X;

}

else {

Y;

}

—--------------------------------

  - So for instance, keeping in line with the example of using hand rotation to control motor speed from before, we could do…

—--------------------------------

if (rHand.theta > 0) {

digitalWrite(L293_INPUT1, HIGH);

digitalWrite(L293_INPUT2, LOW);

}

else {

digitalWrite(L293_INPUT1, LOW);

digitalWrite(L293_INPUT2, HIGH);

}

pwmWrite(L293_ENABLE, PWM_MAXVAL*1.0);

—--------------------------------

And this would make it so if we rotate our hand to the left, the motor spins one way, and spins faster the further we rotate it. If we rotate our hand to the right, it would spin the other way.

Step 7) All that’s left to do is try experimenting with different hand gestures and controlling the motor. You have your table above with the different values you can use, and then the projects so far have taught you how to use them.

What to try next: Try controlling the speed of the motor with your hand! And then if you get that working, can you make it so you control both the direction and speed with different gestures?

Standards Alignment

| Lesson Activity | NGSS Standards | CSTA Standards | ISTE Standards for Students (2016) |
| --- | --- | --- | --- |
| Hardware setup (breadboard, chips, wiring motor & joystick) | MS-ETS1-2: Evaluate design solutions systematically. MS-ETS1-4: Model + test design iterations. PS3.C: Electrical energy transfer into motion. | 2-CS-03: Troubleshoot computing systems. 3A-CS-01: Use abstractions (breadboard as pin extension, libraries like digitalWrite()). | 1. Empowered Learner: Build knowledge by setting up and troubleshooting hardware. 5. Computational Thinker: Model and explore hardware/software systems. |
| Programming the motor (setup & loop functions, pinMode, pwmWrite, digitalWrite) | Science & Engineering Practices: Developing and using models; computational thinking. MS-ETS1-3: Analyze tests to refine motor behavior. | 2-AP-10: Represent algorithms (setup vs loop). 2-AP-13: Decompose problems into subproblems. 3A-AP-17: Modular design. 3A-AP-19: Document/debug code. | 5. Computational Thinker: Apply algorithmic thinking. 4. Innovative Designer: Create and test new code solutions. |
| Adjusting motor speed & direction (hard-coding) | MS-ETS1-3: Analyze performance of different solutions (speed, direction changes). | 2-AP-11: Modify existing programs. 3A-AP-14: Apply control structures (if/else). | 4. Innovative Designer: Refine solutions by iteration. 5. Computational Thinker: Automate solutions through code. |
| Using joystick input to control motor | Science & Engineering Practices: Using math models (mapping 0–1023 joystick values to % motor power). | 3A-AP-16: Incorporate data into program functionality. 3A-AP-23: Evaluate/refine computational artifacts. | 5. Computational Thinker: Translate joystick values into usable program data. 1. Empowered Learner: Personalize computing experiences through input devices. |
| Integrating AI hand-gesture input (ai-motor.c, get_handstate function) | MS-ETS1-2: Evaluate different control approaches (hard-coded vs AI-based). | 3B-IC-27: Analyze computing innovations’ impact on society (gesture control, accessibility) .3A-AP-17: Decompose input → decision → output. | 4. Innovative Designer: Explore novel uses of AI in problem-solving. 6. Creative Communicator: Use AI to extend expression (gesture-based input). |
| Experimenting with gestures to control speed/direction | Science & Engineering Practices: Designing and refining experiments; analyzing system behavior. | 2-AP-19: Document testing/debugging. 3A-AP-22: Design solutions with data-driven decisions. | 1. Empowered Learner: Set learning goals through experimentation. 5. Computational Thinker: Collect and use real-time data (hand gestures → motor response). |
| Bonus exploration: combining joystick + AI for motor control | MS-ETS1-4: Iterative testing of new models and solutions. | 3A-AP-21: Evaluate/refine computational artifacts. 3B-AP-14: Integrate multiple components. | 4. Innovative Designer: Iterate designs with multiple inputs. 7. Global Collaborator: Recognize real-world applications (robotics, accessibility tech). |