RaspPi Lab 2 – Getting Input from the Joystick

Part 1: Setting up the hardware

Connect everything according to the wiring diagram you received with this handout.

Step 1) Put the little chip on the breadboard. Make sure the one you have is labeled MCP3008 (it’s very lightly printed on the top of the chip). You’ll want it to be in the middle, with the pins on either side of the gap of the breadboard. Pay attention to which way the little semi-circular divot on the chip is facing when you put it on the breadboard!

Step 2) Put the power supply on one edge of the breadboard. That’s the black board that goes on the edge of the breadboard with pins that go on either side.

Step 3) Connect the wires according to the wiring diagram you also got with this handout. The colors of the wires don’t matter, but the positions of the wires on the Raspberry Pi and the chip pins do. If a wire on the wiring diagram is shown to be going from pin number 28 on the Pi to the 2nd pin from the left on the top of one of the chips on the breadboard for example, make sure you also have a wire connected in that same way.

- You can basically think of the breadboard as just an extension for the pins of the chips that we plug in. They just make it easier to attach a bunch of wires to the chips without them falling off anytime the chip gets bumped or anything.

Part 2: Programming the Joystick

Now you are going to write a simple program that reads the current value for where the joystick is being pointed.

Step 1) If you remember how from earlier, we are going to open up the terminal and this time open the file called “joystick.c” in the “hotis-projects-starter” folder. If you don’t remember, just follow these steps:

- Open the terminal (the black icon with a little white arrow in the top left of the desktop)

- Type in cd hotis-projects-starter

- Type in git reset --hard

- Type in geany joystick.c

Step 2) The setup function is already done for you in this file. You’ll want to put your code below the line that says // Here you'll want to read a value from the joystick and store it in a variable. Type in the text in green below and you will get a program that can read the joystick value and print that to the screen.

- unsigned xval = analogRead(X_INPUT); This gets the current value of the joystick and stores it in a variable called “xval.” You can then use that variable however you want anytime you need the value of the joystick.

- printf("INFO: analog joystick: x == %4d, y == %4d\n", xval, 0); This line is what prints the value to the screen. The “%4d” parts in this line are formatting to tell the program how many digits to print (4). The “\n” just prints a new line so all of this doesn’t get printed to one line over and over.

Step 4) Now if you compile and run your program, you should be able to move the joystick, and as you move it the value on the screen should update.

- Save the file, then close the editor

- In the terminal, enter make joystick

- Then enter ./joystick

Bonus) You’ll notice something is wrong… For some reason y always stays 0 when you move the joystick around. If you get to this point, try to figure out why and try adding code to also print the y value.

- A quick hint: You’re going to need to figure out which pin on the Raspberry Pi you plugged the y-value wire into, then read from that pin.