RaspPi Lab 3 – Controlling the Motor

Part 1: Setting up the hardware

Yesterday we connected just the joystick to the Raspberry Pi. We are doing that again today, and we are also connecting the motor this time.

Step 1) Put the little chips (there will be 2 of them now) on the breadboard. If you look closely on the chips they have some writing, you want the one on the left to be the one labeled MCP3008, and the one on the right to be labeled L293. Pay attention to which way the little semi-circular divot on the chip is facing when you put it on the breadboard!

Step 2) Put the power supply on one edge of the breadboard. That’s the black board we had connected to a battery from yesterday.

Step 3) Connect the wires according to the wiring diagram you also got with this handout. The colors of the wires don’t matter, but the positions of the wires on the Raspberry Pi and the chip pins do. If a wire on the wiring diagram is shown to be going from pin number 28 on the Pi to the 2nd pin from the left on the top of one of the chips on the breadboard for example, make sure you also have a wire connected in that same way.

- You can basically think of the breadboard as just an extension for the pins of the chips that we plug in. They just make it easier to attach a bunch of wires to the chips without them falling off anytime the chip gets bumped or anything.

Part 2: Programming the motor

Now you are going to write a simple program that turns the motor in a direction at a couple of different speeds.

Step 1) If you remember how from yesterday, we are going to open up the terminal and this time open the file called “motor.c” in the “hotis-projects-starter” folder. If you don’t remember, just follow these steps:

- Open the terminal (the black icon with a little white arrow in the top left of the desktop)

- Type in cd hotis-projects-starter

- Type in geany motor.c

Step 2) In this file there is a line in the “void setup(void)” function that says “ !!!! PUT YOUR SETUP CODE HERE !!!!” Below that line you want to put in the code you need to actually tell the Raspberry Pi which pins are being used as input for the motor, and which pins are being used as output. To do that…

- We use the function pinMode(), which takes two arguments, the pin name and the type of pin we want it to be. You’ll want the following three lines for this section of the file:

- pinMode(L293_ENABLE, PWM_OUTPUT);

- pinMode(L293_INPUT1, OUTPUT);

- pinMode(L293_INPUT2, OUTPUT);

What do these pins actually do? Well, the L293_INPUT1 and L293_INPUT2 pins are actually controlling the direction of the motor. We always set one of these to 1, and the other one to 0. Which we set to 1 and which to 0 determines whether the motor spins clockwise or counterclockwise. The L293_ENABLE pin is what actually tells the motor to spin, and it uses PWM, or pulse-width modulation to tell the motor how fast to spin.

Step 3) Now that we have the setup function done, we can do the loop function. This is where we actually tell the motor to spin. We do this with the functions digitalWrite() and pwmWrite(), both of which we give a pin name and a value to set the pin to. Add the following lines to the loop function below the

!!!! PUT YOUR LOOP CODE HERE !!!! line:

- pwmWrite(L293_ENABLE, PWM_MAXVAL*1.0); This line sets the motor speed to 100%, you can make that lower by changing 1.0 to a different decimal value, for instance, 0.5 would get you 50% power.

- digitalWrite(L293_INPUT1, LOW);

- digitalWrite(L293_INPUT2, HIGH); Lines (b) and (c) here will set the motor to spin forwards. If we wanted it to spin backwards instead, what would you have to change about these two lines?

- delay(1000);

Step 4) Now if you compile and run your program, you should have the motor spin forward at 100% (assuming you left the example code above as-is). You can do that with the following steps:

- Save the file, then close the editor

- In the terminal, enter make motor

- Then enter sudo ./motor

Step 5) If all of that works, then go back into the file and try changing it so the motor spins one way for 5 seconds, then stops and spins the other for 5 more seconds, and then slows down to 60% speed and spins at that from then on.

Bonus) If you want, you’re good to move on to the AI motor project now. However, you could also stop here and try using the joystick to control the motor first. What happens if you use the variable we read from the joystick yesterday to set the speed of the motor? How would you make that variable also set the direction?

- To read the value from the joystick yesterday, we used

int joyVal = analogRead(X_INPUT)

But you may remember from yesterday that got you values that ranged from 0 to 1023. If you want to get an angle instead, you could use the following:

int joyVal = analogRead(X_INPUT) * 30/341 - 45;